IFC_BOX2600 GPIO/WDT Routine

Revision 1.0

- Overview：

This routine is for the Mini PC IFC-BOX2600, altogether there are12 sets GPIO, in which there are 6 GP-O（GP28、GP33、GP34、GP36、GP38、GP39）with 5V TTL level output; the other 6 GP-I(GP9、GP10、GP12、GP13、GP14、GP22) with 3.3V/5V compatible TTL level input (Also these 6 GP-I can be used as output, if so, the output level is 3.3V).

Correspondence:

| Output Type |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Interface S/N. | DO1 | DO2 | DO3 | DO4 | DO5 | DO6 |
| Chipset No. | GP28 | GP33 | GP34 | GP36 | GP38 | GP39 |

| Input Type |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Interface S/N | DI1 | DI2 | DI3 | DI4 | DI5 | DI6 |
| Chipset No. | GP9 | GP10 | GP12 | GP13 | GP14 | GP22 |

Remark: Please don't use these GPIO to run comparatively large load current directly (such as relay,optic coupler etc..)

Besides, it provides a 255sec./min. WDT (Watch Dog Timer).

二、GPIO、WDT Programming Model:

1. a. Set GP-O: Running “GPIO_OUT.EXE”，and set GP28、GP33、GP34、GP36、GP38、GP39 as output, reference code “GPIO_TYPE_OUT.CPP”.

b. Set GP-O as High: Running “HIGHGPIO.EXE”, and set these 6 port as output high, for reference code please refer to: “HIGHGPIO.CPP”.

Set GP-O as Low:  Running “LOWGPIO.EXE”,  and set these 6 ports as output low, for reference code please refer to: “LOWGPIO.CPP”.

Remark: Please use multimeter or indicator lights to verify the  status of “High” or “Low” level output.

2. Set GP-I: Running “GPIO_IN.EXE”, and set GP9、GP10、GP12、GP13、GP14、GP22 as input, for reference code please refer to “GPIO_TYPE_IN.CPP”

Remark: Running “GETIO” to verify the status of “High” or “Low” level input.